NN      NN  EEEEEEEEEE  WW      WW  TTTTTTTTTT

NNNN    NN  EE          WW      WW      TT

NNNN    NN  EE          WW      WW      TT

NN  NN  NN  EEEEEE      WW  WW  WW      TT

NN    NNNN  EE          WW  WW  WW      TT

NN    NNNN  EE          WW  WW  WW      TT

NN      NN  EEEEEEEEEE    WW  WW        TT

NEWT - a graphical program to quickly try various design

parameters in designing a NEWTonian telescope.

Version 2.0

Copyright 1994 by Dale A. Keller

Page 1 of 15

Requirements:

- An IBM PC or compatible with an EGA or VGA display.

NEWT will NOT work with CGA or Hercules MDA.  The shading

and aspect ratio is much better on VGA than EGA.

- A Microsoft compatible mouse.

Options:

- Any text printer.

Description:

NEWT is a Newtonian telescope CAD (Computer Aided Design)

program.  It uses a GUI (Graphical User Interface) to design

a Newtonian telescope, and allows you to modify all the

parameters.  Capabilities include:

- Display a side view of a newtonian telescope.

- Modify all parameters like mirror diameter, diagonal

minor axis, focuser height, focuser inside diameter,

tube inside diameter and thickness, and many more.

- Use inches, centimeters, or millimeters as the unit of

measure.

- Calculate baffle size and placement.

- Calculate diagonal offset.

- Calculate fields of 100% and 75% illumination.

- Load and Save files.

- Eyepiece table includes focal length, power, exit pupil,

apparent field, and true field.

- Check for minimum useable diagonal minor axis.

- Check for vignetting at the focuser.

- Check for vignetting of true field by top of tube.

- Calculate obstruction percentages for light loss and

contrast reduction.

- Cast three rays through telescope - an on-center ray,

a 100% light zone ray, and a 75% light zone ray.  Use

these to check for vignetting at focuser, true field

cutoff, etc.

- Print specifications & dimensions to a generic text

printer, or a text file.

- GUI driven - easy to use.

- Built in screen saver.

Price:

Use NEWT as long as you like.  If you want to pay for it, the

price is $10.00.  If you don't want to pay, enjoy it anyway.

If you don't enjoy it, let me know why and I might fix it.  If

you do enjoy it, give a copy to someone else.

I've always thought shareware should have a maximum price of

$10.00 no matter how good the program is, and that there should

be no requirement to "pay if you use it over 30 days".  I think

most people would pay 10 bucks, even for software that isn't

perfect.  How many shareware programs have you had that cost 50

to 80 dollars that were pretty good, but not quite worth invest-

ing in?  Would you pay $10.00 for that same program?

NEWT is not public domain software.  NEWT is copyright (c)

Dale A. Keller, 1994.

Page 2 of 15

==============================================================

How to run NEWT

==============================================================

The only file required to run NEWT is NEWT.EXE.  NEWT can

also use telescope parameter files with the extension .NWT.

Several sample .NWT files are included.

Simply type NEWT at the DOS prompt.  There are no command line

parameters.

A graphical display will appear, showing the default telescope.

The default is a run-of-the-mill 6 inch F 8.  At the top of the

display is a menu, the scope is drawn in the middle, and a set

of messages is shown at the bottom.

The easiest way to find out what NEWT does is to play with it.

Then, when you want to know some detail about something, look it

up in this manual.

The telescope parameter files are plain ASCII and can be edited

by any text processor.  They must remain in the format produced

by NEWT, however, to be loadable.  Since you can modify parameters

using NEWT and the save a new file, it is probably safest to just

let NEWT edit these files for you.

The parameter files supplied are:

WHITE.NWT - a generic 6" f8 - not optimized

BLACK.NWT - a 4.25" f4 - not optimized

RED.NWT - a 6" f11 - a good planetary scope

BLUE.NWT - a 10" f5.6 - a good deep-sky scope

These files are telescopes the author owns and can be used as

examples.  They aren't perfect, but they are closer than they

were before using this program.  The WHITE and BLACK telescopes

are pretty generic, and show a couple of the problems with non-

optimized designs.

Page 3 of 15

===============================================================

USING NEWT

===============================================================

Using the Mouse:

To select an item:

Place the mouse pointer on a button or filename and click

the left mouse button.

To cancel or close a window:

Click the right mouse button anywhere, or click the left

mouse button on the CANCEL button (when available).

Using the Keyboard:

To accept an edited field:

Press Enter.

To reject an edited field:

Press Esc.

To move to the next field (on Specs and Eyepieces windows):

Press Tab instead of Enter.  This only works when a field

is being edited.

The Main Screen:

When NEWT starts, it displays a row of menu buttons across

the top of the screen, the current ray-traced telescope in

the middle, and a message box at the bottom.

The rays cast through the telescope are color coded.  The

on-center ray is green, the ray at the edge of the 100%

illuminated zone is red, and the ray at the edge of the

75% illuminated zone is yellow.

The message box at the bottom of the screen displays comments

about the adequacy of the diagonal size to admit the cone of

light for the 100% zone, and the detection of vignetting of

the light cone.  Problems are highlighted in red.  Items which

are adequate are printed in black.

To correct problem areas, use the Specs menu to alter the

telescope variables.  More will be said about this in the

"Notes on Telescope Design" section below.

Page 4 of 15

===============================================================

The Menu Selections

===============================================================

-   File:

Load:

Loads a selected telescope parameter file.

Notes: To change directories, click on the directory

you want to go to, and then click on ACCEPT.

Save:

Saves the current telescope variables to a parameter

file.

Notes: To change directories, click on the directory

you want to go to, and then click on ACCEPT.

About NEWT:

Displays the copyright notice and the version number.

-   Specs:

Displays the current telescope variables and allows you

to edit them.

Notes: All variables must be in a common unit of measure.

The buttons near the bottom labeled "in", "cm",

and "mm" are INCH, CENTIMETER, and MILLIMETER.

Press Tab instead of Enter to move the cursor to

the next field.

-   Eyepieces:

Displays the set of eyepieces you will use with this

telescope (up to 10) and allows you to edit them.

Notes: All focal lengths must be in millimeters.

Apparent and true fields are in degrees.

Press Tab instead of Enter to move the cursor to

the next field.

-   View:

Zoom in on focuser:

Displays a closeup of the focuser to better see the

ray tracing in this critical area.  The 100% zone of

the focal plane is the white horizontal line.  The

75% zone is the gray line.

Baffle list:

Displays a list of the positions and diameters of the

baffles.

Performance info:

Displays a list of expected performance specifications

for the current telescope.

Dimensions:

Displays a list of calculated dimensions to assist in

fabrication of the current telescope.

-   Options:

Show on-center rays:

Displays on-center rays (green) when checked.

Page 5 of 15

Show 100% zone rays:

Displays 100% zone rays (red) when checked.

Show 75% zone rays:

Displays 75% zone rays (yellow) when checked.

Screen Saver:

If checked, screen saver is enabled.

Screen Saver Timeout:

Number of minutes of no keyboard or mouse activity

until screen saver is activated.

-   Print:

To Text Printer:

Prints the telescope variables, specifications, and

dimensions to a text printer.

To File:

Prints the telescope variables, specifications, and

dimensions to a text file named x.OUT where x is the

name of the current telescope file (x.NWT).

-   Quit:

Exits the program.

Page 6 of 15

==============================================================

Design Parameters

==============================================================

Specifications:

Primary Diameter: The diameter of the primary mirror.

Focal Ratio: The focal length of the primary mirror divided

by the diameter.

Tube Inside Diameter: The inside diameter of the telescope

tube.

Tube Thickness: The thickness of the telescope tube walls.

Focuser Height: The height of the top of the focuser above

the telescope tube wall.

Spare Focuser In Travel: The distance from the top of the

focuser tube in the "in" position to the focal plane.

Note: Some books say the focal plane should coincide

with the top of the focuser tube. However, many eye-

pieces will NOT come to focus this way. See the "Notes

on Telescope Design" section below.

Additional Height For Camera: The amount of extra distance

from the focal plane required to reach the film in a

camera.  Use zero if you are not using a camera.

Focuser Inside Diameter: The inside diameter of the focuser.

Diagonal Minor Axis: The distance across the diagonal mirror

on it's narrowest face.

Focuser To Front Of Tube: The amount of tube from the center

of the focuser hole to the end of the telescope tube.

Fixed Diameter Baffles: A switch to force all the baffles to

be the same size as the front baffle.  If not used, the

baffle diameters will follow the 75% light cone.

Unit Of Measure: All variables must be in a common unit of

measure, Inches, Centimeters, or Millimeters.

Eyepieces:

Eyepiece (mm): The focal length of the eyepiece in millimeters.

Power: The computer-calculated power for the eyepiece when used

in the current telescope.

Exit Pupil: The computer-calculated diameter of the exit pupil

for this eyepiece when used in the current telescope.

Apparent Field: The apparent field of this eyepiece in degrees.

Page 7 of 15

True Field: The computer-calculated true field of the sky when

viewed through this eyepiece in the current telescope.

Dimensions:

Note: Most of the dimensions listed in the VIEW/DIMENSIONS

window are the same as the Specifications variables

listed above. Therefore, only the dimensions calculated

by the computer will be listed here.

Focal Length: The distance from the primary mirror to where

it focuses, calculated from the primary diameter and the

focal ratio.

Secondary Offset: The distance from the center of the tube

toward the tube wall opposite the focuser that the diag-

onal mirror should be positioned.

100% Illumination Diameter: The diameter of the focal plane

which is fully illuminated by the primary mirror.

75% Illumination Diameter: The diameter of the focal plane

which is 3/4 illuminated by the primary mirror.

Front Aperture Diameter: The diameter of the light cone as

it passes through the front of the telescope tube (for

the 75% illuminated field).  This is the diameter of the

front baffle.

Mirror Face To Focuser Hole: The distance from the front

surface of the primary mirror to the center of the

focuser hole.

Page 8 of 15

==============================================================

Notes on Telescope Design

==============================================================

This is not indended to be a thorough dissertation on Newtonian

telescope design.  See the references on the last page for more

information.

Many of the calculations done by NEWT were taken from standard

formulas in books.  Some were invented out of necessity by the

author.  In some cases, more than one formula was available,

giving somewhat different results.  And of course, every

telescope builder has his or her opinion of the "correct" way to

do something.  If you have suggestions to "correct" or improve

NEWT, please contact the author.

The purpose of a telescope is to collect light from a distant

object, bring it to a clear and undistorted focus, magnify it,

and deliver it to a detector (the eye, a camera, etc.).

Many factors can interrupt or distort the flow of light in a

telescope, degrading the image quality.  That is the reason for

NEWT - to quickly find the problem areas in a design, adjust

variables, and check the results.

Some of the most common problem areas are:

Vignetting - a reduction of the amount of light reaching

the focal plane by objects intruding into the light

path.

Glare - stray light reflected in the telescope which inter-

feres with the light from the object of interest.

Improper Design - mismatched optics which do not allow the

full light cone to reach the focal plane unimpeded.

Atmospheric Distortion - moving or variable-temperature air

above or inside the telescope which distorts the image.

We will look at each of these problem areas in order.

Vignetting:

Vignetting occurs when the cone of light strikes an object

in it's path before it reaches the eyepiece.  The most

common problem area is the inside diameter of the focuser.

To see where the light cone might strike the focuser, use

the "Zoom in on Focuser" option in the VIEW menu.

A tall focuser increases the probability of vignetting. A

narrow focuser does the same thing.  Many telescopes are

built with the standard 1-1/4 inch by 3-1/2 inch focuser.

At any focal ratio less than about F 10, this can cut off a

fair amount of the light cone.  Using a low profile, wide

focuser is one of the easiest ways to improve a telescope.

Page 9 of 15

Even if the 75% zone light cone just passes through the

focuser, it can help to alter the focuser.  What the eye-

piece sees is all of the 100% zone, all of the 75% zone, and

then a very sharp falloff of light to zero.  No 50% zone is

available at all.  This will effect a low power, wide angle

eyepiece the most, because this eyepiece has a very large

field lens.  If the lens is wider than the 75% zone, it will

be very dark near the edges of the field of view.

Another place vignetting occurs is at the front end of the

tube.  Notice in the example file WHITE.NWT that the 75% ray

strikes the tube wall near the front.  This means that the

75% zone is reduced in size, and there is a sharp cutoff of

light at it's edge.  To solve this problem, increase the

diameter of the tube.  See the example file "WHITE.NWT" for

a 6 inch F8 telescope (this is the default telescope which

is displayed when NEWT is started).

Glare:

A very common problem in newtonian telescopes is glare. An

easy way to check a telescope for strong glare is to view a

star near the moon, but with the moon just outside of the

field of view.  The light from the moon, shining on the

inside of the tube wall, bounces around trying to get into

the observers eye.  This stray light brightens the whole

field, reducing contrast.  The same thing happens even when

the telescope is pointed away from the moon - city lights,

bright planets, and even fairly faint sky glow can get into

places it shouldn't and reduce contrast.

One way to reduce glare is to install baffles in the tube.

If these baffles closely match the desired cone of incoming

light, and are spaced properly, then most of the stray light

coming from other directions will hit a baffle instead of

finding it's way to the eyepiece.  A rough, very flat black

surface inside the tube walls will also help to soak up

stray light.

Note: Baffles which closely match the light cone can some-

times cause another problem - tube current eddies.

See "Atmospheric Distortion" below.

Adding baffles to a telescope can sometimes improve image

contrast enough to allow the observer to detect very faint

details which could not be seen before.

There are many ways to make baffles.  One simple suggestion

is to use foam weatherstripping for sealing doors -

available at any hardware store.  The foam should have a

rough surface to soak up light.  Fortunately, this type of

foam is less expensive than the higher density, slick type.

The tape backing will not hold up under the temperature

range a telescope is used in, so it should be glued in

place.  A couple coats of very flat black paint should be

applied.

Page 10 of 15

The algorithm used by NEWT to calculate the position and

diameter of the baffles only works for the lower end of the

telescope.  NEWT adds one baffle at the top, and one baffle

on either side of the focuser.  In actual practice, several

baffles should be placed above and below the focuser to

create as much shadow as possible on the inside tube wall

opposite the focuser.

Improper Design:

To deliver as much light as possible to the focal plane, the

primary mirror, diagonal mirror, and all other components

must match.  Too small a diagonal will produce a very small

or non-existent zone of 100% illumination.  Too large a

diagonal will block light to the primary mirror and reduce

contrast.  A compromise must be reached between 100% light

zone size and contrast loss.

A telescope designed for planetary and high power use will

usually use shorter focal length eyepieces.  These eyepieces

have a fairly small field lens and don't need a large 100%

zone.  Only the central area of the field needs to be fully

illuminated, because that is where the object of interest is

placed for viewing.  The focal ratio of the telescope is

also fairly high, F 10 or more, so the light cone is very

narrow.  Therefore a small diagonal mirror can be used which

will not reduce the contrast very much.  High contrast is

very desirable for planetary detail.  See the example file

"RED.NWT" for a 6" F11 planetary telescope.

A telescope designed for deep sky, low power use will often

use longer focal length eyepieces.  These eyepieces have a

larger field lens, and need a correspondingly larger 100%

illuminated zone.  The entire field should be illuminated as

much as possible, so objects near the edge of the field of

view are not dim.  This requires care to keep the 75% and

50% zones from being vignetted.  A larger diagonal mirror

should be used.  However, image contrast is still very

important for seeing faint detail in extended objects. Using

too large a diagonal can produce larger zones at the cost of

degrading image quality.  See the example file "BLUE.NWT"

for a 10" F5.6 deep-sky telescope.

Some sizes of telescope are just hard to make work.  A small

diameter primary mirror and a short focal ratio make a poor

telescope.  The short focal ratio wants a larger diagonal

and a low profile focuser, but the focuser does not shrink

in proportion with the tube size.  There is also a limit on

how much the clearance can be reduced between the edge of

the primary mirror and the tube walls.  These two factors

cause a large percent of the light cone to be used up

reaching from the diagonal to the focal plane.  This forces

the use of a larger diagonal.  See the example file

"BLACK.NWT" for a 4-1/4 inch F4 telescope.

Page 11 of 15

Atmospheric Distortion:

Much has been said in books and magazines on reducing air

currents above and inside a telescope.  The only idea

covered here regards the design of the light baffles. In an

open tube design, there will always be tube currents. Some

telescope makers use an oversize tube to allow the

inevitable currents to stay near the walls, out of the light

path.  Baffling a tube to reduce glare can interfere with

this current, causing the air to flow into the light path as

it moves past the baffle.  This can cause severe image

distortion, wavy images, double images, and other problems.

To solve this problem, the baffles can be made fairly

shallow, and placed closer together.  This allows the air

moving near the tube walls to flow closer and stay out of

the light path.   NEWT has an option (on the SPECS menu) to

use fixed diameter baffles.  When this option is on, all the

baffles will be the same diameter as the front baffle. The

front baffle diameter is designed to be the same diameter as

the 75% zone cone of light as it passes through the front of

the telescope.

----------------------------------------------------------------

Notes on Illumination Size:

General:

The focal plane is generally fully illuminated in the

center, and gradually tapers off in brightness toward

the edge.  A common way of measuring the illuminated

area is by defining the zone of full illumination (the

100% zone), and the area where the brightness has

tapered off to 75%.

The 100% zone is the area at the focal plane which is

fully illuminated by the primary mirror.  This area will

have 100% of the brightness available from the primary

mirror.  This is the area produced by the light cone

from the primary, reflected from the diagonal, as long

as there is no vignetting.  Changing the diagonal minor

axis is the easiest way to change the size of this zone.

The 75% zone is the area at the focal plane which is 3/4

illuminated by the primary mirror.  This area will be

dimmer than the 100% area, tapering off in brightness

from the edge of the 100% zone until only 75% of the

brightness from the primary mirror is available at the

edge of the 75% zone.

Visual Use:

An eyepiece will usually have approximately the same

field lens diameter as its focal length.  So to fully

illuminate the field of a 12 mm eyepiece, a 12 mm (1/2

inch) area of 100% illumination is required.  Full

illumination is not absolutely required, and in fact

usually drops off to around 75% near the edges of the

eyepiece field.

Page 12 of 15

The larger the eyepiece field lens, the larger areas of

100% and 75% illumination required.  This is also

impacted by the diagonal mirror minor axis and any

possible vignetting by other elements of the telescope,

such as the focuser inside diameter.

Some practical limit must be reached, however, because

increasing the diagonal size will also decrease

contrast and light gathering ability.  One possible rule

of thumb is to limit the size of the 100% zone to 1/2 of

the field lens size of the largest eyepiece expected to

be used.

Photographic Use:

Generally, to attain the brightest image (and utilize

the full potential of the telescope's light gathering

ability), the film in the camera should be as fully

illuminated as possible.  This will require a sub-

stantially larger diagonal mirror than does visual

work.

In a 35mm camera, the short dimension of the film is

24mm (about 1 inch).  The camera body requires the focal

plane to be moved farther out from the focuser as well.

Adding 2 inches of focal plane height for the camera

body, and requiring a 1 inch area of 100% illumination

will call for a fairly large diagonal mirror.

The other components of the telescope must be redesigned

to accomodate photographic work.  The focuser inside

diameter must be larger to prevent vignetting of the

light cone, and the diagonal mirror spider mount must

be strong enough to prevent the heavier mirror from

vibrating or sagging.

A telescope which is optimized for photographic use does

not usually perform well for visual work.

Notes on Spare Focuser In Travel:

This is the distance from the top of the focuser tube (when

racked all the way in) to the focal plane (where the light

from the primary mirror comes to a focus).  Some eyepieces

focus farther in than others.  Shorter focal lengths usually

need to be racked farther in than longer ones.  Note: If you

will be using the telescope for terrestrial viewing, you

will need some extra "out" travel to focus on objects which

are closer than the heavenly bodies.  You will usually have

more spare "out" travel than "in" travel.

The amount of "Spare Focuser In Travel" should usually be

about 1/4 to 1/2 inch.

One way to set up a telescope right on the first try (and to

avoid drilling focuser holes and mirror mount holes all over

Page 13 of 15

the place) is to design the scope with about 1/2 inch spare

travel.  Then, using the calculated measurements for

component placement, mount the diagonal mirror and the

focuser and place an eyepiece in it.  Mount the primary

mirror in it's mirror cell, and get an assistant to slide

the mirror into the calculated position in the tube.  Try to

focus on a VERY distant object.  The assistant should slide

the primary mirror in and out until the object comes into

focus.  Mark this position, remove the optics, and then

drill the holes for the primary mirror mount.  Note: Using

an object that is not far enough away to focus upon will

give an improper result.  An object at least 1/2 mile

distant would be the minimum.

Another way to get the proper Spare Travel is to measure

where each eyepiece's focal plane lies in relation to it's

seating position in the focuser.  The field stop in an

eyepiece is usually at the eyepiece's focal plane.  This

focal plane must coincide with the primary mirror's focal

plane for an image to be in focus.  Measure the distance

from the bottom of the eyepiece tube (the part that slides

into the focuser) to the field stop.  Then measure the

distance from the bottom of the eyepiece tube to where the

eyepiece stops in the focuser (the top of the part that

slides into the focuser).  If the field stop sits down in

the focuser tube, you will have to rack the focuser out to

reach focus.  If the field stop sits above the focuser tube,

you MUST add some extra "in" travel to move the eyepiece in

far enough to focus.  The amount to add is the distance of

the field stop above the focus tube, plus any safety margin

desired.

Notes on Using a Camera:

If you will only use the telescope visually, use Zero for

the variable "Additional Height for Camera".  If you will be

using a camera, enter the amount of additional height for

the focal plane above the focal plane used for eyepieces.

This is usually about 2 inches for 35 mm Single Lens Reflex

cameras.  You should also add any height required for an

off-axis guider.  This is usually three quarters to one

inch.

This will increase the probability of vignetting the light

cone, because a larger conic section will have to fit

through the focuser and other parts of the telescope to

reach the focal plane.  For this reason, a telescope

designed for camera use should have the lowest and widest

possible focuser.

The size of the diagonal mirror should also be taken into

account.  A larger diagonal may be required to accomodate

the 100% zone desired.  This will decrease contrast and

block more light from the primary mirror.  A telescope

should be designed for either visual or photographic work if

possible, although a dual purpose scope can be built with

some compromises.  See "Notes on Illumination Size".

Page 14 of 15

==============================================================

References

==============================================================

A couple of good (cheap) books on telescope design are:

All About Telescopes by Sam Brown

from Edmund Scientific

Newtonian Notes by Peter Francis

from Kenneth Novak & Co.

There are many others, mostly at a higher cost, that are very

good.  There are some, also at a higher cost, that are a waste

of money.  You'll have to discern for yourself.

==============================================================

About the Author

==============================================================

NEWT version 2.0 is copyrighted by Dale A. Keller (1994) and is

distributed as SHAREWARE.

Please send comments, questions, suggestions, and money to:

Dale A. Keller

3056 W. Pimlico Dr.

Englewood, CO. 80110

Page 15 of 15